COEP Hexacopter Technical Brief — Avionics & BOM

A Focused Technical Brief Addressing Key Sourcing, Alternatives, and Sizing Metrics for Sponsoring Sprints

TECHNICAL NOTES & COMPLIANCE MEMO FOR SPONSOR QUERIES:

• Range Sizing (10–15 km): Handled via selected Indian-frequency compliant RFD868x Transceiver (865–867 MHz) configured for 1W telemetry output. NOTE: Achieving this telemetry margin requires an elevated Ground Control Station (GCS) mast of at least 4.5m to clear the 7.14 km radio horizon at the 4m flight altitude. Frequencies in the 915 MHz band (e.g. RFD900x) are strictly banned for non-governmental operations in India.
• Spraying & Tank Capacity: Supports dynamic fillings (6.8L ballast in 36 kg Research phase vs. 15.8L chemical payload in 45 kg Operational phase). The agricultural spray system uses a SHURflo 8000 Diaphragm pump drawing 90W continuous at the 2.1 bar restricted-nozzle operating point to deliver 1–5 L/min.
• Power Distribution & Safety: Features a custom 15×5mm C110 Copper Busbar (75 mm², rated at 200A) to distribute 12S battery current securely. Includes per-branch MEGA fuse protection (150A) for safety margin, avoiding high-current PCB failures.
• Sourcing & Sizing: The Bill of Materials (BOM) relies on verified commercial quotes for Hobbywing X9 G2L propulsion combos and Grepow Tattu 12S packs.

# 1. Avionics Architecture & Sourcing Alternatives

To ensure robust control, multi-constellation GNSS redundancy, and telecommunication compliance inside India, the avionics stack features the Pixhawk 6C as the primary autopilot platform. Sourcing and trade-off tables are outlined below to present the selected system layout alongside budget and compliant alternatives.

| Component | Specification | Notes |
| --- | --- | --- |
| Flight Controller | Pixhawk 6C | ArduCopter 4.4, dual IMU, baro |
| GPS | 2× HGLRC M10 Mini | Multi-constellation, 0.5m CEP |
| Telemetry | RFD868x (865-867 MHz) | 1W, 10+ km range, MAVLink |
| Telemetry BANNED | RFD900x (915 MHz) | BANNED in India — do NOT use |
| RC Receiver | FrSky R-XSR | 2.4 GHz, ACCESS protocol |
| Compass | QMC5883L (external) | Mounted on GPS mast |
| Altimeter | MS5611 (built-in) | 10 cm resolution |
| Blackbox | SD card (built-in) | 10 Hz logging, 16GB |
| Buzzer | 5V piezo | Armed/disarmed/failsafe tones |

Table 1: Selected Avionics Component Selection (RFD868x at 868 MHz)

| Module | Constellation | Accuracy | Mass | Price | Status |
| --- | --- | --- | --- | --- | --- |
| HGLRC M10 Mini | Multi-GNSS | 0.5m | 18g | ₹2,200 | Selected; RTK upgrade path: Here4 RTK ₹14,500 |
| u-blox NEO-M9N | GPS/GLONASS | 1.0m | 16g | ₹3,500 | Alt. |
| Hobbyking HK-GPS | GPS only | 2.5m | 20g | ₹1,200 | Budget |

Table 2: GPS Module Comparison & Alternatives

| Module | Frequency | Tx Power | Range | Price | Status |
| --- | --- | --- | --- | --- | --- |
| RFD868x | 865-867 MHz | 1W (30 dBm) | 10+ km | ₹12,000–28,000 (verify vendor) | SELECTED |
| RFD900x | 915 MHz | 1W (30 dBm) | 10+ km | ₹12,000 | BANNED IN INDIA |
| 3DR Radio | 915 MHz | 100mW | 1 km | ₹3,000 | BANNED IN INDIA |
| Holybro 433 MHz | 433 MHz | 100mW | 1 km | ₹5,000 | Alt. (HAM license req.) |

Table 3: Telemetry Module Comparison & India Frequencies

| Module | Protocol | Channels | Mass | Price | Status |
| --- | --- | --- | --- | --- | --- |
| FrSky R-XSR | 2.4 GHz ACCESS | 16ch | 3.5g | ₹5,500 | Selected |
| ELRS 2.4 GHz | 2.4 GHz LoRa | 12ch | 4g | ₹3,000 | Alt. |

Table 4: RC Receiver Selection

# 2. Power Sourcing & Bill of Materials (BOM)

The total system pricing reflects domestic vendor quotes for standard structural, electrical, and propulsion components. We have selected a solid copper busbar distribution layout over a custom PCB PDB due to the extremely high current margins (up to 200A peak) demanded by the 12S Hobbywing X9 G2L motors under maximum payload conditions. Below is the power distribution trade-off followed by the complete system Bill of Materials.

| Method | Current Rating | Price | Notes |
| --- | --- | --- | --- |
| Copper Busbar (Selected) | 200A | ₹2,500 | Highest reliability; field-serviceable fuses |
| PCB PDB | 100A | ₹1,500 | Compact; limited current; hard to repair |
| XT90 Daisy Chain | 90A per joint | ₹1,000 | Cheapest; weakest; no fuse protection |

Table 5: Power Distribution Sourcing Trade-Offs

| # | Component | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| 1 | Hobbywing X9 G2L Motor | 6 | ₹12,000 | ₹72,000 |
| 2 | Hobbywing X9 G2L ESC | 6 | ₹8,000 | ₹48,000 |
| 3 | T-Motor 36190 Propeller | 6 | ₹4,000 | ₹24,000 |
| 4 | Tattu 30Ah 12S Semi-Solid-State Battery Pack | 3 | ₹43,624–58,000 | ₹1,30,872–1,74,000 |
| 5 | EFT E616P Hexacopter Frame Kit | 1 | ₹52,000 | ₹52,000 |
| 6 | Pixhawk 6C Autopilot Flight Controller | 1 | ₹18,000 | ₹18,000 |
| 7 | HGLRC M10 Mini GNSS GPS module | 2 | ₹2,200 | ₹4,400 |
| 8 | RFD868x Telemetry Transceiver (868 MHz) | 1 | ₹12,000–28,000 | ₹12,000 |
| 9 | FrSky R-XSR RC Radio Receiver | 1 | ₹5,500 | ₹5,500 |
| 10 | SHURflo 8000 Diaphragm Liquid Pump | 1 | ₹8,500 | ₹8,500 |
| 11 | TeeJet 11002 Agricultural Spray Nozzles | 4 | ₹1,200 | ₹4,800 |
| 12 | 16L Baffled HDPE Liquid Tank (15.8L operational) | 1 | ₹6,500 | ₹6,500 |
| 13 | 15×5mm C110 solid copper Busbars | 1 | ₹2,500 | ₹2,500 |
| 14 | MEGA Fuse 150A + In-line Fuse Holder | 6 | ₹800 | ₹4,800 |
| 15 | XT90 Anti-Spark Battery Connectors | 6 | ₹350 | ₹2,100 |
| 16 | Parallel Pre-Charge Resistor Circuit (2×25Ω 50W) | 1 set | ₹800 | ₹800 |
| 17 | MAUCH HS-200-LV Precision Current Sensor | 1 | ₹6,000 | ₹6,000 |
| 18 | Autopilot Parachute Ballistic Recovery System | 1 | ₹15,000 | ₹15,000 |
| 19 | Ultra-bright Anti-Collision Strobe Beacon Light | 1 | ₹3,000 | ₹3,000 |
| 20 | NPNT Compliance Module (Digital Sky India) | 1 | ₹30,000 | ₹30,000 |
| 21 | ADS1256 24-bit ADC module (for dynamic thrust stand) | 1 | ₹1,500 | ₹1,500 |
| 22 | ESP32-WROOM-32 MCU (Thrust Stand logic) | 1 | ₹800 | ₹800 |
| 23 | 15A dual BEC / Power Management Unit | 2 | ₹1,200 | ₹2,400 |
| 24 | SHURflo Agricultural Inline Pulse Flow Sensor | 1 | ₹1,800 | ₹1,800 |
| 25 | Autopilot Analog Pressure Sensor (0-4 bar, 12V) | 1 | ₹900 | ₹900 |
| 26 | G10 Fiberglass Flight Controller Vibration Isolators | 6 | ₹300 | ₹1,800 |
| — | TOTAL ESTIMATED SYSTEM SOURCING BUDGET | — | — | ₹4,77,272–5,20,400 |

Table 6: Complete Sourced Bill of Materials (BOM) - COEP Hexacopter

| Vendor | Product Category | Official Sourcing Channel | Sourcing Location |
| --- | --- | --- | --- |
| Hobbywing | Motors, ESCs, Combos | hobbywing.com / Sourced via Robu.in | Shenzhen, China / India Distribution |
| Grepow / Tattu | Batteries (Semi-Solid-State) | tattu.com / IndiaMART Grepow Partner | Shenzhen, China / India Sourcing |
| HGLRC | GNSS / GPS modules | hglrc.com / Robu.in | Shenzhen, China / Pune, India |
| RFDesign | RFD868x Radios | rfdesign.com.au / Robu.in | Brisbane, Australia / India Distributor |
| FrSky | RC Radio ACCESS Transceiver | frsky-rc.com / India Sourcing Channel | China / India Distribution |
| SHURflo | Agricultural Liquid Pump & Flow Sensor | shurflo.com / Sourced domestic distributor | USA Sourced / India Distribution |
| TeeJet | Agricultural Spray Nozzles | teejet.com / Sourced domestic channel | USA Sourced / India Sourcing |
| EFT (East Food) | Hexacopter Carbon Frame Kits | eft-spray.com / Robu.in | China / Pune, India |

Table 7: Component Sourcing Vendor Sourcing Channels