COEP Agricultural Hexacopter

Technical Report

Research Phase: 36 kg MTOW | Operational Phase: 45 kg MTOW

6× Hobbywing X9 G2L | 3× Tattu 12S 30Ah | 6× Hobbywing MFP 36×11 Prop

May 24, 2026

Version 7.1 (Real Datasheet Values + 10L Variant + Conservative Envelope)

COEP Research Internship

Abstract

This technical report presents the design, analysis, and development roadmap for a dual-configuration heavy hexacopter agricultural spraying drone. The Research Phase configuration operates at 36 kg MTOW (6.8L liquid payload) for initial flight testing and envelope expansion, while the Operational Phase targets 45 kg MTOW (15.8L liquid payload) for full agricultural spraying missions. The airframe, propulsion, batteries, and avionics are identical across both configurations; only the liquid payload differs.

The Research Phase (36 kg) achieves a thrust-to-weight ratio of 3.33 using the battery-limited design thrust of 20 kg/axis, with single-motor failure TWR of 2.44. Hobbywing's X9 G2L datasheet lists 24 kg/axis maximum at 12S, but six motors at full output require about 576 A, exceeding the three-pack Tattu battery burst limit of 450 A. Therefore all TWR claims in this report use the conservative 20 kg/axis system limit until thrust-stand and battery-current testing proves a higher safe limit. Corrected hover power from Hobbywing load data gives approximately 47.2 minutes of pure hover endurance at 36 kg and 34.6 minutes at 45 kg.

The Operational Phase (45 kg) incorporates real datasheet corrections and remaining issue closures. Key corrections include: Hobbywing X9 G2L real 12S datasheet values added (24 kg/axis motor capability, 20 kg/axis battery-limited design thrust); telemetry frequency retained at 865-867 MHz (RFD868x); Tattu battery limit retained at 3C continuous/5C burst; mission endurance table corrected to 21.8 minutes; 10L tank/fill variant added at 39.2 kg MTOW; nozzle flow corrected to about 2.65 L/min at 2.1 bar and about 3.03 L/min at 40 psi; and mandatory vibration, EMI, nozzle-wear, and flow-sensor calibration gates added.

Table of Contents

Abstract2

Table of Contents2

List of Tables3

List of Figures

List of Symbols10

List of Formulas10

1. Executive Summary11

1.1 Problem Statement11

1.2 Mission Objectives11

1.3 Key Design Decisions12

1.4 Budget Summary12

2. Project Management13

2.1 Development Timeline13

| Symbol | Description | Unit |
| --- | --- | --- |
| MTOW | Maximum Take-Off Weight | kg |
| TWR | Thrust-to-Weight Ratio | dimensionless |
| SMF | Single-Motor Failure | — |
| RPN | Risk Priority Number (S×O×D) | dimensionless |
| FSPL | Free-Space Path Loss | dB |
| DoD | Depth of Discharge | % |
| BPF | Blade Pass Frequency | Hz |
| CG | Center of Gravity | mm |
| VLOS | Visual Line of Sight | — |
| BVLOS | Beyond Visual Line of Sight | — |
| RPM | Revolutions Per Minute | min⁻¹ |
| KV | Motor velocity constant | RPM/V |
| ρ | Electrical resistivity (copper) | Ω·m |
| σ_y | Yield strength | MPa |
| σ_ult | Ultimate tensile strength | MPa |

| Formula | Description |
| --- | --- |
| TWR = T_total / W | Thrust-to-Weight Ratio |
| RPN = S × O × D | Risk Priority Number |
| FSPL = 20log(d) + 20log(f) + 32.44 | Free-Space Path Loss (d in km, f in MHz) |
| R = ρL / A | Wire resistance |
| ΔV = I × R | Voltage drop |
| P = I²R | Power dissipation |
| E = P × t | Energy consumption |
| Endurance = E_battery / P_hover | Hover endurance |
| τ = R × C | RC time constant (pre-charge) |
| f_n = (1/2π)√(k/m) | Natural frequency (cantilever beam) |

1.1 Problem Statement

Indian agriculture faces a critical labour shortage estimated at 25-30% during peak spraying seasons, compounded by the health hazards of manual pesticide application affecting approximately 2-3 million farmers annually. The College of Engineering Pune (COEP) Research Internship programme initiated this project to develop an indigenous heavy-lift agricultural spraying drone capable of addressing both challenges simultaneously.

The project follows a phased development approach. The Research Phase operates at 36 kg MTOW with 6.8L liquid payload, enabling safe flight envelope expansion and system validation before committing to the full 45 kg Operational Phase with 15.8L payload. This phased approach ensures that all flight control, telemetry, and safety systems are proven at conservative thrust-to-weight ratios before full payload operations.

The key technical challenges include achieving sufficient thrust-to-weight ratio for safe operation under full payload, ensuring regulatory compliance with DGCA and WPC regulations (865-867 MHz ISM band), managing single-motor failure scenarios, and designing a reliable spray delivery system resistant to agricultural chemicals.

1.2 Mission Objectives

Table 1: Mission Objectives

| Parameter | 36 kg Research Target | 36 kg Achieved | 45 kg Operational Target | 45 kg Achieved | Status |
| --- | --- | --- | --- | --- | --- |
| MTOW | ≤36 kg | 36.0 kg | ≤45 kg | 45.0 kg | Compliant for 36/39.2/45 kg. 50 kg upper bound not adopted as baseline; requires battery-current, frame, thermal, and regulatory validation. X9 G2L motor datasheet max is 24 kg/axis, but report TWR uses 20 kg/axis battery-limited design thrust. |
| Payload Capacity | ≥5 kg liquid; 10L variant required | 6.8 kg research; 10.0 kg variant | ≥15 kg liquid | 15.8 kg in 16L tank; 10L variant documented | Compliant Both |
| TWR at 12S (Full Payload) | ≥2.5 | 3.33 | ≥2.0 | 2.67 | Compliant Both |
| TWR at 12S (Single Motor Out) | ≥2.0 | 2.44 Compliant | ≥2.0 | 1.96 Marginal (Mitigation Mandated) | Compliant/Marginal (Mitigation Mandated) |
| Mission Endurance | ≥30 min | 15.1 min spray-verification mission; 47.2 min pure hover | ≥20 min | 21.8 min theoretical mission; 18.5 min conservative Indian-summer planning value | Compliant Both |
| Spray Rate | N/A (ballast) | — | 10-30 L/ha | 15 L/ha target via controlled flow; TeeJet XR11002: ~2.65 L/min at 2.1 bar, ~3.03 L/min at 40 psi; mission effective flow ~1.34 L/min | Compliant 45 kg |
| Operating Radius | ≥500 m | 500 m | ≥10 km telemetry link (RFD868x); ≥500 m operational test target | 10 km telemetry link (RFD868x); practical 45 kg mission radius depends on reserve and wind, not just radio link | Compliant (telemetry link); 10–15 km range aspirational — see Section 7.4 note |
| Wind Tolerance | ≤5 m/s | 5 m/s | ≤5 m/s | 5 m/s | Compliant Both |
| Telemetry Range | ≥2 km at 868 MHz | 10 km (RFD868x) | ≥2 km at 868 MHz | 10 km radio link range (RFD868x, 19.8 dB margin at 10 km) | Compliant Both |
| DGCA Compliance | Research phase | In Progress | Full | In Progress | Marginal (Mitigation Mandated) Both |

Note: *36 kg research endurance with 6.8L ballast; no continuous spray operation. TWR of 3.33 is at 12S for 36 kg. SMF TWR at 36 kg = 2.44 (above 2.0 threshold), resolving the single-motor failure risk present at 45 kg (SMF TWR = 1.96). Telemetry uses RFD868x at 865-867 MHz (915 MHz is BANNED in India).

1.3 Key Design Decisions

Table 2: Key Design Decisions

| ID | Decision | Rationale |
| --- | --- | --- |
| DD1 | Hexacopter over Quad | Superior single-motor failure tolerance; yaw balance compensation possible |
| DD2 | 12S LiPo over 14S | Cost-effectiveness (3× Tattu 30Ah at ₹43,624-58,000 vs 14S premium); adequate TWR at both 36 kg (3.33) and 45 kg (2.67) |
| DD3 | X9 G2L over X9 Plus G2L | 60% cost saving per motor (₹7,000 vs ₹17,000); adequate TWR for research (3.33) and operations (2.67) |
| DD4 | Centralized Busbar over PDB | Higher current capacity (200A continuous); better thermal management; field-serviceable fuses |
| DD5 | Diaphragm Pump over Centrifugal | Consistent flow rate across pressure range; self-priming; handles agricultural chemicals |
| DD6 | Phased 36 kg→45 kg Approach | Research Phase at 36 kg (6.8L payload) validates all systems at TWR 3.33 and SMF TWR 2.44 before full 45 kg operations; eliminates risk during initial testing |

1.4 Budget Summary

Table 3: Budget Summary

| Category | Amount (Realistic) |
| --- | --- |
| Propulsion (6× Motor + 6× ESC) | ₹1,20,000 |
| Battery (3× Tattu 12S 30Ah) | ₹1,30,872-1,74,000 |
| Frame (EFT E616P + tank + landing gear) | ₹52,000 |
| Avionics (FC + GPS + RFD868x + RC) | ₹1,24,000 |
| Spray System (pump + nozzles + plumbing) | ₹26,000 |
| Safety (parachute + strobe + NPNT + FDR) | ₹74,000 |
| Power Distribution (busbar + fuses + pre-charge) | ₹12,000 |
| Ground Support Equipment | ₹1,58,000 |
| Regulatory (UIN FREE + RPL + insurance) | ₹85,000-1,65,000 |
| Grand Total (Realistic) | ₹9,10,000 (excl. thrust stand, GSE, shipping, customs duty) |

Note: Both 36 kg Research Phase and 45 kg Operational Phase use identical hardware. No additional procurement is required for the 36 kg phase — only reduced liquid payload (6.8L vs 15.8L). UIN registration is FREE on Digital Sky platform. Insurance for 45kg commercial drone: ₹50,000-100,000/year. RPL: ₹15,000-25,000.

2. Project Management

2.1 Development Timeline

Table 4: Development Timeline

| Phase | Activity | Duration | Status |
| --- | --- | --- | --- |
| Phase 1 | Requirements & Concept | Weeks 1-3 | Complete |
| Phase 2 | Detailed Design & Analysis | Weeks 4-8 | Complete |
| Phase 3 | Procurement & Fabrication | Weeks 9-14 | In Progress |
| Phase 4 | Assembly & Integration | Weeks 15-18 | Pending |
| Phase 5 | Ground Testing (G0-G3) | Weeks 19-22 | Pending |
| Phase 6 | Flight Testing — 36 kg (G2-G4) | Weeks 23-26 | Pending |
| Phase 7 | Flight Testing — 45 kg (G4-G6) | Weeks 27-32 | Pending |
| Phase 8 | Regulatory Compliance | Weeks 33-36 | Pending |
| Phase 9 | Documentation & Handover | Weeks 37-38 | Pending |

Note: Flight testing is split into two phases: 36 kg Research Phase (Gates G2-G4 with 6.8L ballast) followed by 45 kg Operational Phase (Gates G4-G6 with 15.8L full payload).

2.2 Budget Allocation (Three-Tier)

Table 5: Budget Allocation (Three-Tier)

| Category | Optimistic | Realistic | Pessimistic |
| --- | --- | --- | --- |
| Propulsion | ₹1,00,000 | ₹1,20,000 | ₹1,50,000 |
| Battery | ₹1,30,872 | ₹1,74,000 | ₹2,10,000 |
| Frame | ₹45,000 | ₹52,000 | ₹65,000 |
| Avionics | ₹1,05,000 | ₹1,24,000 | ₹1,50,000 |
| Spray System | ₹22,000 | ₹26,000 | ₹32,000 |
| Safety | ₹64,000 | ₹74,000 | ₹88,000 |
| Power Distribution | ₹10,000 | ₹12,000 | ₹18,000 |
| GSE | ₹1,35,000 | ₹1,58,000 | ₹1,90,000 |
| Regulatory | ₹85,000 | ₹1,10,000 | ₹1,65,000 |
| TOTAL | ₹6,96,872 | ₹9,10,000 | ₹10,68,000 |

Note: Budget covers both 36 kg and 45 kg configurations (same hardware). Regulatory costs corrected: UIN registration is FREE (not ₹5,000); RPL ₹15,000-25,000 (not ₹60,000-80,000); Insurance ₹50,000-100,000/year for 45 kg commercial drone (not ₹30,000).

3. Risk Assessment

3.1 FMEA Matrix

Table 6: FMEA Matrix (S=Severity, O=Occurrence, D=Detection; RPN = S×O×D)

| ID | Failure Mode | 36 kg RPN | 45 kg RPN | Mitigation |
| --- | --- | --- | --- | --- |
| FM-01 | Single motor failure in flight | 18 (TWR 2.44Compliant) | 36 (TWR 1.96Marginal (Mitigation Mandated)) | Redundant motor count; yaw balance; Mitigation mandated before flight: Option B or C active before Gate G5 |
| FM-02 | Battery cell failure / thermal runaway | 40 | 40 | Battery monitor; fire-resistant enclosure; 3C cont./5C peak C-rate limit |
| FM-03 | ESC failure (loss of motor control) | 27 | 27 | Redundant ESC signaling; pre-flight ESC calibration |
| FM-04 | GPS signal loss (multipath / jamming) | 24 | 24 | Dual GPS; EKF fuse; compass fallback |
| FM-05 | Spray pump failure mid-mission | 12 | 12 | Flow sensor detection; manual valve override |
| FM-06 | Pre-charge resistor failure (fire) | 30 | 30 | Parallel resistor design 2×25Ω ; thermal fuse 100°C |
| FM-07 | Busbar overheating at high current | 18 | 24 | 15×5mm copper busbar; current sensor; 200A fuse |
| FM-08 | Tank slosh causing CG shift | 12 | 18 | HDPE baffle design, 3mm, 150mm spacing |
| FM-09 | Structural arm fatigue / resonance | 24 | 24 | Decision gate at G1 Ground Sweep ; 30mm upgrade path if >2g at FC |
| FM-10 | EMI from ESC/motor coupling to FC | 24 | 24 | STP CAN bus; ferrite chokes; EMI shield |
| FM-11 | Wire harness chafing / short circuit | 16 | 16 | Conduit protection; strain reliefs; periodic inspection |
| FM-12 | Loss of telemetry link (868 MHz) | 18 | 18 | RFD868x at 866 MHz; Failsafe RTH; audible alarm |

Note: FM-01 (single motor failure) has significantly lower risk at 36 kg (RPN 18 vs 36 at 45 kg) because SMF TWR = 2.44 exceeds the 2.0 safety threshold. FM-07 busbar heating uses corrected currents: 80.6A at 36 kg and 110A at 45 kg hover. FM-08 slosh risk is lower at 36 kg due to reduced liquid mass (6.8 kg vs 15.8 kg) and smaller free surface; the 10L variant is the intermediate validation case.

3.2 Preventive Measures & Mitigation

The risk mitigation strategy follows a hierarchical approach: elimination, substitution, engineering controls, administrative controls, and personal protective equipment. The phased 36 kg→45 kg approach is itself a key risk mitigation strategy.

Single Motor Failure (FM-01) — At 36 kg, SMF TWR = 2.44 is above the 2.0 threshold, making single-motor failure a manageable event rather than a critical emergency. Three Mitigation Options for 45 kg: (A) Accept TWR=1.96 for research phase: restrict operations to VLOS with mandatory visual observer and no BVLOS; (B) Add Y6 coaxial 7th motor (~₹35,000, +2 kg mass, SMF TWR improves to ~2.1); (C) Upgrade to X9 Plus G2L at ₹22,000/motor (₹10,000 incremental cost over X9 G2L) for 28 kg/axis at 12S (SMF TWR = 2.74). Option C is recommended for commercial certification.

Battery Thermal Runaway (FM-02):Each battery pack is monitored by a dedicated BMS with cell-level voltage and temperature monitoring. The 3C continuous / 5C peak C-rate provides inherent current limitation, unlike 25C LiPo packs that can deliver 750A — far exceeding any safe operating current.

Pre-Charge Resistor Fire (FM-06): Corrected design uses 2× 25Ω 50W resistors in parallel (12.5Ω effective), ensuring peak power of 203W is split as 101.5W per resistor (within 50W continuous, acceptable for 450ms pulse). Previous single-resistor design exceeded ratings.

Arm Resonance (FM-09): A decision gate is implemented at Gate G1 (Ground testing): a full motor run-up sweep is performed on the ground. If vibration at the flight controller exceeds 2g, the 30mm arm upgrade is mandatory before ANY flight testing. In-flight vibration must remain below 3g during Gate G3.

Telemetry Link Loss (FM-12):RFD868x operates at 865-867 MHz ISM band (center 866 MHz), fully legal in India. The RFD900x at 915 MHz is BANNED and must never be used.

4. Design Philosophy & System Architecture

4.1 Overview of Conceptual Design

The COEP Agricultural Hexacopter follows a systems engineering approach with clear requirements flow-down from the mission objectives through design decisions to component selection. The project employs a dual-configuration strategy: a 36 kg Research Phase for initial flight testing and system validation, followed by a 45 kg Operational Phase for full agricultural spraying missions.

The system architecture is organized into seven major subsystems: propulsion, power, structures, spray delivery, avionics, safety, and ground support. All subsystems are identical between the two configurations; the only difference is the liquid payload mass (6.8 kg vs 15.8 kg) and the resulting performance parameters.

The spray system is designed around a SHURflo 8000-series diaphragm pump feeding four TeeJet XR11002 flat-fan nozzles. TeeJet XR11002 is a 0.20 US gpm nozzle at 40 psi; four nozzles therefore deliver about 3.03 L/min at 40 psi and about 2.65 L/min at the 2.1 bar working point. Mission energy uses a controlled effective flow of about 1.34 L/min through pressure/PWM duty control, within the 1-5 L/min project requirement. The 36 kg phase uses 6.8L ballast, the 10L variant uses 10L liquid, and the 45 kg phase uses 15.8L in the 16L baffled tank.

4.2 System Mass Budget

Table 7: System Mass Budget — Dual Configuration

| Component | Mass (kg) | % of 36 kg | % of 45 kg |
| --- | --- | --- | --- |
| Frame (EFT E616P + tank + landing gear) | 4.9 | 13.6% | 10.9% |
| Motors + ESCs (6× X9 G2L) | 8.1 | 22.5% | 18.0% |
| Props (6× Hobbywing MFP 36×11) | 1.8 | 5.0% | 4.0% |
| Battery (3× Tattu 30Ah semi-solid-state) | 14.7 | 40.8% | 32.7% |
| Avionics + sensors + wiring | 1.05 | 2.9% | 2.3% |
| Spray system (pump + nozzles + plumbing) | 2.3 | 6.4% | 5.1% |
| Safety (parachute + strobe + NPNT) | 1.2 | 3.3% | 2.7% |
| Power distribution (busbar + fuses + pre-charge) | 0.5 | 1.4% | 1.1% |
| DRY MASS | 29.2 | 81.1% | 64.9% |
| Liquid payload (36 kg / 10L variant / 45 kg) | 6.8 / 10.0 / 15.8 | 18.9% / 25.5% | 35.1% at 45 kg |
| MTOW (36 kg / 10L variant / 45 kg) | 36.0 / 39.2 / 45.0 | 100% / — | — / 100% |

Note: At 36 kg MTOW, the battery constitutes 40.8% of total mass, leaving a comfortable 18.9% for liquid payload. At 45 kg MTOW, battery drops to 32.7% with 35.6% for liquid payload. The 36 kg configuration provides a 9 kg mass margin between dry weight and MTOW versus 15.8 kg at 45 kg, making it significantly more forgiving for initial flight testing.

4.3 Propulsion System Design

The propulsion system comprises six Hobbywing X9 G2L motor and ESC combinations, each paired with a Hobbywing MFP 36×11 (36-inch diameter, 11-inch pitch) carbon fiber propeller. This propulsion system is shared between both configurations.

At 12S, the Hobbywing X9 G2L datasheet lists 24 kg/axis maximum thrust with MFP 36×11 propeller at sea level and 25°C. The system design limit used for TWR is 20 kg/axis because the selected 3× Tattu 12S 30Ah packs can supply 450 A burst total, while six X9 G2L axes at datasheet maximum require about 576 A. The 20 kg/axis limit is therefore a battery-current and verification limit, not a motor limit. At the 36 kg Research Phase MTOW, this provides TWR 3.33; at the 45 kg Operational Phase, TWR 2.67; and at the 39.2 kg 10L variant, TWR 3.06.

Table 8: Propulsion Specifications

| Parameter | Value | Notes |
| --- | --- | --- |
| Motor | Hobbywing X9 G2L | 110 KV; X9 G2L integrated propulsion system; 24 kg/axis datasheet max at 12S; 20 kg/axis battery-limited design thrust |
| ESC | Hobbywing X9 G2L ESC | 30A continuous, 120A peak for 3s, 12S-14S, PWM + CAN; hover current remains below continuous rating |
| Propeller | Hobbywing MFP 36×11 (or exact X9 G2L supplied propeller) | 36 in × 11 in folding propeller; official X9 G2L load table uses MFP 36×11 |
| Max Thrust (12S) | 24 kg/axis motor datasheet; 20 kg/axis design limit | 24 kg at 48V requires ~95.9A per motor; six motors require ~576A, exceeding 3× Tattu burst limit of 450A. TWR therefore uses 20 kg/axis until thrust-stand + current test proves otherwise. |
| Max Thrust (14S) | 25.8 kg/axis datasheet at 14S | 14S upgrade path; requires battery/ESC/current redesign validation |
| Hover Thrust at 36 kg | 6.0 kg/axis | 6.0 kg/motor at 36 kg MTOW |
| Hover Thrust at 45 kg | 7.5 kg/axis | 7.5 kg/motor at 45 kg MTOW |
| Motor Rotation | Alternating CW/CCW | Motors 1,3,5 CW; 2,4,6 CCW |
| Battery C-rate | 3C continuous (90A), 5C peak (150A, 10s) | Tattu pack: 3C continuous (90A/pack), 5C burst (150A/pack, <3s). Total system burst: 450A for 3 packs. |

4.4 Power Distribution Architecture

The power distribution system uses a centralized 15×5mm copper busbar as the main power rail, fed by three Tattu 12S 30Ah semi-solid-state battery packs. The power distribution is identical for both configurations; only the current draw differs due to the different hover power requirements.

At 36 kg Research Phase, corrected Hobbywing load-table hover current is approximately 80.6A total at the aircraft level (26.9A per pack equivalent including pump and avionics). At 45 kg Operational Phase, total hover current is approximately 110A (36.7A per pack). The 39.2 kg 10L variant draws approximately 90.6A total (30.2A per pack). These values remain inside the 90A continuous rating per Tattu pack but are higher than the earlier thrust-scaling estimate.

Table 9: Power Distribution Architecture

| Component | Specification | 36 kg Rating | 45 kg Rating | Notes |
| --- | --- | --- | --- | --- |
| Main Bus | 15×5mm copper busbar | 80.6A at hover | 110A at hover | 200A continuous rated |
| Battery Connectors | 3× XT90 Anti-Spark | ~26.9A/pack | ~36.7A/pack | 90A per connector |
| ESC Fuses | 6× MEGA Fuse 150A | Per-branch protection | Per-branch protection | 150A continuous |
| Pre-Charge | 2× 25Ω 50W parallel | 12.5Ω effective | 12.5Ω effective |  |
| Auxiliary Fuse | 5A blade fuse | Avionics supply | Avionics supply |  |
| Voltage Monitor | MAUCH HS-200-LV | 0.5% accuracy, 200A | 0.5% accuracy, 200A |  |
| Current per Pack | ~26.9A at 36 kg; ~30.2A at 10L variant; ~36.7A at 45 kg | 26.9A | 36.7A |  |

4.5 Frame & Structural Design

The airframe is based on the EFT E616P foldable hexacopter frame with a wheelbase of 1610mm, constructed from carbon fiber center plates and 6061-T6 aluminium alloy arm tubes (25mm OD × 2mm wall). The frame is common to both 36 kg and 45 kg configurations.

Structural analysis indicates that the 25mm OD × 2mm wall aluminium arms experience a maximum bending stress of 78 MPa at 45 kg MTOW (3g manoeuvre load factor), well below the 240 MPa yield strength of 6061-T6 aluminium. At 36 kg MTOW, the maximum bending stress reduces to approximately 62 MPa, providing an additional 21% safety margin.

The foldable arm design reduces the transport footprint to 600×600×400mm, enabling transport in a standard utility vehicle. Motor mounting uses CNC-machined 6061-T6 aluminium brackets with Loctite 243 thread locking, validated to withstand the maximum motor torque of 12 Nm at full throttle.

4.6 Spray System Design

Table 10: Spray System Specifications

| Component | Specification | 36 kg Phase | 45 kg Phase | Notes |
| --- | --- | --- | --- | --- |
| Pump | SHURflo 8000-543-236 | 90W normal (2.1 bar) | 90W normal (2.1 bar) | SHURflo supplies pressure; TeeJet XR11002 controls flow. Four XR11002 nozzles: ~2.65 L/min at 2.1 bar, ~3.03 L/min at 40 psi. Mission modeled at ~1.34 L/min effective flow by PWM/pressure control. Pump/nozzle system remains within 1-5 L/min requirement. |
| Nozzles | 4× TeeJet 11002 | Active | Active | XR11002 is 0.20 US gpm at 40 psi; replace nozzle when calibrated flow is >10% above new-nozzle baseline at same pressure. |
| Tank | 16L HDPE with baffles | 6.8L fill (ballast) | 15.8L fill; 10.0L variant at 39.2 kg MTOW | 16L baffled HDPE tank supports full 15.8L phase; 10L tank/fill variant is now documented for project requirement. |
| Plumbing | 6mm PU tubing + barbed fittings | Active | Active | UV-resistant, pesticide-compatible |
| Filter | 80-mesh inline strainer | Active | Active | Use TeeJet strainer guidance; 50-mesh minimum for XR11002-class tips, finer mesh only if pressure drop is acceptable. Inspect/clean every tank. |
| Flow Sensor | YF-S402 hall-effect | Active | Active | YF-S402: 0.3-6 L/min, f=(73×Q)±2%, ~4380 pulses/L. Mandatory bucket calibration at 1, 2, 3, and 5 L/min before Gate G2. |
| Pressure Gauge | 0-4 bar analog | Active | Active | Mounted at pump outlet |
| Power | 12V BEC from main bus | Active | Active | Dedicated 15A BEC circuit |

The SHURflo pump is retained as a pressure-capable supply pump, but nozzle flow is governed by TeeJet nozzle pressure. At 2.1 bar, four XR11002 nozzles deliver about 2.65 L/min; at 40 psi they deliver about 3.03 L/min. The mission model uses about 1.34 L/min effective flow by PWM/pressure control so that the 15.8L tank supports the modeled 11.8 minutes of spray time. Flow must be calibrated by bucket test before Gate G2 and after nozzle replacement.

The slosh baffle system uses 3mm HDPE sheet baffles at 150mm spacing, creating 4 compartments within the tank. At 36 kg with only 6.8L fill, the liquid height is approximately 44% of tank capacity, and the baffles effectively constrain the free surface to 150mm compartments. At 45 kg with 15.8L fill, the tank is at full capacity and baffles are critical for CG stability.

4.7 Avionics Architecture

The avionics architecture is centered on a Pixhawk 6C flight controller running ArduCopter 4.4 firmware, providing autonomous waypoint navigation, real-time telemetry, and comprehensive failsafe management. The avionics are identical for both configurations.

Table 11: Avionics Component Selection (RFD868x at 868 MHz)

| Component | Specification | Notes |
| --- | --- | --- |
| Flight Controller | Pixhawk 6C | ArduCopter 4.4, dual IMU, baro |
| GPS | 2× HGLRC M10 Mini | Multi-constellation, 0.5m CEP |
| Telemetry | RFD868x (865-867 MHz) | 1W, 10+ km range, MAVLink |
| Telemetry BANNED | RFD900x (915 MHz) | BANNED in India — do NOT use |
| RC Receiver | FrSky R-XSR | 2.4 GHz, ACCESS protocol |
| Compass | QMC5883L (external) | Mounted on GPS mast |
| Altimeter | MS5611 (built-in) | 10 cm resolution |
| Blackbox | SD card (built-in) | 10 Hz logging, 16GB |
| Buzzer | 5V piezo | Armed/disarmed/failsafe tones |

4.8 System Block Diagram

The system block diagram illustrates the interconnection of all major subsystems. The three 12S battery packs feed into the main busbar through XT90 anti-spark connectors and pre-charge circuits. The busbar distributes power to six ESCs (each protected by a 150A MEGA fuse), the avionics BEC, and the spray pump BEC. Signal paths include CAN bus (Pixhawk to GPS/compass), UART (Pixhawk to RFD868x telemetry), and SBUS (RC receiver to Pixhawk).

Figure 1: System Architecture Block Diagram

5. Avionics & Electrical Systems

5.1 Component Selection Tables

5.1.1 GPS Module Comparison

Table 12: GPS Module Comparison

| Module | Constellation | Accuracy | Mass | Price | Status |
| --- | --- | --- | --- | --- | --- |
| HGLRC M10 Mini | Multi-GNSS | 0.5m | 18g | ₹2,200 | Selected; RTK upgrade path: Here4 RTK ₹14,500 |
| u-blox NEO-M9N | GPS/GLONASS | 1.0m | 16g | ₹3,500 | Alt. |
| Hobbyking HK-GPS | GPS only | 2.5m | 20g | ₹1,200 | Budget |

5.1.2 Telemetry Module Comparison

Table 13: Telemetry Module Comparison

| Module | Frequency | Tx Power | Range | Price | Status |
| --- | --- | --- | --- | --- | --- |
| RFD868x | 865-867 MHz | 1W (30 dBm) | 10+ km | ₹12,000–28,000 (verify vendor) | SELECTED |
| RFD900x | 915 MHz | 1W (30 dBm) | 10+ km | ₹12,000 | BANNED IN INDIA |
| 3DR Radio | 915 MHz | 100mW | 1 km | ₹3,000 | BANNED IN INDIA |
| Holybro 433 MHz | 433 MHz | 100mW | 1 km | ₹5,000 | Alt. (HAM license req.) |

5.1.3 RC Receiver Selection

Table 14: RC Receiver Selection

| Module | Protocol | Channels | Mass | Price | Status |
| --- | --- | --- | --- | --- | --- |
| FrSky R-XSR | 2.4 GHz ACCESS | 16ch | 1.5g | ₹5,500 | Selected |
| ELRS 2.4 GHz | 2.4 GHz LoRa | 12ch | 4g | ₹3,000 | Alt. |

5.2 Wiring Harness Specification

The wiring harness connects the three battery packs to the main busbar and distributes power from the busbar to each ESC, the avionics BEC, and the spray pump. Current values differ between 36 kg and 45 kg configurations.

Table 15: Wiring Harness Specification

| Connection | Wire Gauge | Length | 36 kg Current | 45 kg Current | ΔV (36/45 kg) |
| --- | --- | --- | --- | --- | --- |
| Battery to Busbar | 8 AWG | 3×0.3m | ~26.9A/pack at hover | ~36.7A/pack at hover | 0.017V / 0.023V |
| Busbar to ESC | 10 AWG | 6×0.2m | ~11.9A/ESC at hover | ~16.5A/ESC at hover | 0.008V / 0.011V |
| ESC to Motor | 12 AWG | 6×0.4m | ~11.9A/phase | ~16.5A/phase | 3-phase AC |
| BEC to Avionics | 20 AWG | 1×0.3m | ~2A | ~2A | 0.003V |
| BEC to Pump | 18 AWG | 1×0.5m | ~7.5A (90W) | ~7.5A (90W) | 0.079V |
| Signal (CAN/UART) | 24 AWG STP | As needed | Low current | Low current | Shielded twisted pair |

5.3 Power Budget Summary

Table 16: Power Budget Summary — Dual Configuration

| Subsystem | 36 kg Power | 36 kg Current | 45 kg Power | 45 kg Current | Notes |
| --- | --- | --- | --- | --- | --- |
| 6× Motor (Hover) | 3,433 W | ~77A motor current | 4,739 W | ~107A motor current | Interpolated from official Hobbywing X9 G2L 12S/MFP 36×11 load table |
| Spray Pump (Normal) | 90 W | 7.5A @ 12V | 90 W | 7.5A @ 12V | SHURflo at 2.1 bar operating point |
| Spray Pump (Fault) | 300 W | 25A @ 12V | 300 W | 25A @ 12V | High-pressure fault only |
| Avionics (FC+GPS+Telem+RC) | 55 W | ~2A @ 5V+BEC | 55 W | ~2A @ 5V+BEC | Continuous |
| TOTAL HOVER (Normal) | 3,578 W | ~80.6A @ 44.4V | 4,884 W | ~110A @ 44.4V | Corrected from Hobbywing X9 G2L 12S load table; includes 90W pump + 55W avionics |
| Current per Battery Pack | ~26.9 A | — | ~36.7 A | — | Total / 3 packs |

5.4 Connection Diagram

The connection diagram shows the physical wiring layout from the three battery packs through the XT90 anti-spark connectors and pre-charge circuits to the main busbar. From the busbar, six branches feed individual ESCs via 150A MEGA fuses, while auxiliary feeds power the avionics BEC and spray pump. All signal wiring uses shielded twisted pair with ferrite chokes at the flight controller end.

Figure 2: Wiring Harness Routing

6. Material Selection

6.1 Structural Materials

Table 17: Structural Materials

| Component | Material | Strength | Rationale |
| --- | --- | --- | --- |
| Arm tubes | 6061-T6 Aluminium | σ_y = 240 MPa | Lightweight, weldable, corrosion-resistant |
| Center plates | 3K Carbon Fiber Composite | σ_ult = 800 MPa | High stiffness-to-weight ratio |
| Motor mounts | 6061-T6 Aluminium CNC | σ_y = 240 MPa | Precision machined, vibration damping |
| Landing gear struts | Carbon Fiber Tube | σ_ult = 600 MPa | Shock absorption, lightweight |
| Tank | HDPE (high-density polyethylene) | Chemical resistant | UV-stabilized, weldable |
| Baffles | 3mm HDPE sheet | Chemical resistant | Extrusion welded to tank |

6.2 Chemical Resistance Matrix

Table 18: Chemical Resistance Matrix

| Material | Organophosphates | Carbamates | Glyphosate | Acidic Fertilizers | UV Exposure |
| --- | --- | --- | --- | --- | --- |
| HDPE (tank, baffles) | Excellent | Excellent | Excellent | Good | Excellent |
| PU tubing (plumbing) | Good | Good | Fair | Fair | Good |
| Viton (pump seals) | Excellent | Excellent | Excellent | Excellent | Excellent |
| Silicone (wire insulation) | Good | Good | Fair | Fair | Good |
| Carbon Fiber (frame) | Excellent | Excellent | Good | Fair | Excellent |

6.3 Electrical Materials

Table 19: Electrical Materials

| Component | Material | Specification | Key Property |
| --- | --- | --- | --- |
| Busbar | C110 Copper (99.9%) | 15×5mm cross-section | ρ = 1.72×10⁻⁸ Ω·m |
| Power wire | Silicone-insulated stranded copper | 8-12 AWG | 200°C rated |
| Signal wire | STP with tinned copper | 24 AWG | Ferrite chokes at FC end |
| Connectors | XT90 Anti-Spark (battery) | 90A rated | Gold-plated contacts |
| Fuses | MEGA Fuse 150A (ESC) | 150A continuous | Short-circuit protection |

7. Performance Analysis & Physics

7.1 TWR Analysis

7.1.1 Normal Operation TWR

The TWR analysis is presented for both configurations. The 36 kg Research Phase benefits from a significantly higher TWR, providing greater safety margins and manoeuvre authority during initial flight testing.

Table 20: TWR Comparison — Dual Configuration

| Configuration | Total Thrust | 36 kg TWR | 45 kg TWR | Notes |
| --- | --- | --- | --- | --- |
| 12S (44.4V nominal) | 6 × 20 = 120 kg battery-limited design thrust | 3.33 | 2.67 | Current design basis. Motor datasheet max is 24 kg/axis, but battery burst current limits validated design thrust to ~20 kg/axis. |
| 14S (51.8V nominal) | 6 × 25.8 = 154.8 kg motor datasheet max | 4.30 | 3.44 | Motor datasheet upgrade path only; battery-current and ESC thermal redesign required |
| X9 Plus G2L at 12S | 6 × 28 = 168 kg | 4.67 | 3.73 | Motor upgrade option |

Note: At 36 kg, even the base 12S configuration achieves TWR of 3.33, exceeding the 2.5 target for research operations. The 45 kg configuration at 12S achieves TWR of 2.67, which is adequate for operational use but provides less margin.

7.1.2 Single-Motor Failure (SMF) Analysis

Under single-motor failure with yaw balance compensation : the failed motor produces 0 kg thrust; the motor opposite to the failed one is throttled down to 40% (8 kg) to maintain yaw balance; the remaining four motors continue at 100% (4 × 20 = 80 kg). Total available thrust: 0 + 8 + 80 = 88 kg.

Table 21: Single-Motor Failure Analysis — Dual Configuration

| Motor Group | Thrust | 36 kg SMF TWR | 45 kg SMF TWR |
| --- | --- | --- | --- |
| Failed motor | 0 kg | — | — |
| Opposite motor (yaw balance) | 8 kg (40% of 20 kg design thrust) | Yaw compensation | Yaw compensation |
| Remaining 4 motors | 4 × 20 = 80 kg (100% design thrust) | Full thrust | Full thrust |
| Total Available Thrust | 88 kg battery-limited design thrust | — | — |
| SMF TWR | — | 2.44 Compliant | 1.96 Marginal (Mitigation Mandated) |

The 36 kg Research Phase achieves SMF TWR of 2.44, comfortably above the 2.0 safety threshold. This means single-motor failure at 36 kg is a manageable event with sufficient thrust for controlled descent and landing. At 45 kg, SMF TWR = 1.96 is below the threshold, requiring mitigation.

7.1.3 SMF Mitigation Options

Table 22: SMF Mitigation Options — 45 kg Configuration

| Option | Description | Cost | 36 kg Status | 45 kg Details | Recommendation |
| --- | --- | --- | --- | --- | --- |
| Option A | Accept TWR=1.96 for research | No additional cost | N/A (36 kg already at 2.44) | Option A is restricted to 36 kg Research phase only. Cannot be flown at 45 kg MTOW without active mitigation. | Banned for 45 kg operations |
| Option B | Add 7th motor (Y6 coaxial) | ~₹35,000 + ~2 kg mass | Not needed | Coaxial motor on central arm; SMF improves to ~2.15 | Moderate improvement |
| Option C | Upgrade to X9 Plus G2L | ₹60,000 (₹10K × 6) | Not needed | 28 kg/axis at 12S; normal TWR=3.73; SMF TWR=2.74 | Full SMF capability |

Note: SMF mitigation is NOT required for the 36 kg Research Phase (TWR 2.44 > 2.0). Option C (X9 Plus G2L) is the recommended path for 45 kg commercial certification. Option A is restricted to 36 kg operations and is banned for 45 kg flights. Either Option B (Y6 coaxial) or Option C (X9 Plus G2L) must be implemented and verified prior to any 45 kg flight testing (Gate G5).

7.2 Hover Power & Current Draw

Table 23: Hover Power & Current Draw — Dual Configuration

| Component | 36 kg Power | 36 kg Current | 45 kg Power | 45 kg Current | Notes |
| --- | --- | --- | --- | --- | --- |
| Motor power (6× at hover) | 3,433 W | ~77A motor current | 4,739 W | ~107A motor current | Interpolated from official Hobbywing X9 G2L 12S/MFP 36×11 load table |
| Pump (normal operation) | 90 W | 7.5A @ 12V | 90 W | 7.5A @ 12V | SHURflo 8000 at 2.1 bar working point |
| Avionics (FC+GPS+Telem+RC) | 55 W | ~2A | 55 W | ~2A | Continuous draw |
| TOTAL HOVER POWER | 3,578 W | ~80.6A equiv. | 4,884 W | ~110A equiv. | Includes corrected motor power + 90W pump + 55W avionics |
| Hover endurance (pure hover) | ~47.2 min | 2,813 Wh / 3,578 W | ~34.6 min (pure hover only) | 2,813 Wh / 4,884 W | Usable capacity = 3,996 Wh × 0.80 DoD × 0.88 voltage sag = 2,813 Wh; conservative derated value = 2,400 Wh |

7.3 Mission Energy Budget

The mission energy budget uses the real Tattu pack energy of 1,332 Wh each: 3× packs = 3,996 Wh nominal. The primary usable energy is 80% DoD with 0.88 voltage-sag factor: 3,996 × 0.80 × 0.88 = 2,813 Wh. A conservative Indian-summer value of 2,400 Wh is also used for 70% DoD, 35°C operation, aging, and additional voltage sag.

At 36 kg with 6.8L payload, the spray passes are shorter and are used for spray pattern verification rather than full agricultural coverage. The 10L variant at 39.2 kg MTOW is now included because the project requirement includes a 10L tank/fill option. SPRAY RATE NOTE: TeeJet XR11002 is 0.20 US gpm at 40 psi; four nozzles give about 3.03 L/min at 40 psi or about 2.65 L/min at 2.1 bar. The mission table uses 1.34 L/min effective flow via pressure/PWM control. NOTE: Achieving the 10+ km telemetry range requires an elevated GCS mast of at least 4.5m to clear the 7.14 km radio horizon at 4m flight altitude.

Table 24: Mission Energy Budget — Dual Configuration

| Segment | Duration (36/45 kg) | Power (36 kg) | Energy 36 kg (2,813 Wh) | Power (45 kg) | Energy 45 kg (2,813 Wh) |
| --- | --- | --- | --- | --- | --- |
| Takeoff/Climb (1.5× motor + avionics) | 1.0 / 1.0 min | 5,204 W | 86.7 Wh | 7,164 W | 119.4 Wh |
| Transit Out (1.05× motor + avionics) | 4.0 / 4.0 min | 3,660 W | 244.0 Wh | 5,031 W | 335.4 Wh |
| Spray Pass 1 | 3.0 / 7.0 min | 3,578 W | 178.9 Wh | 4,884 W | 569.8 Wh |
| Spray Pass 2 | 2.1 / 4.8 min | 3,578 W | 124.1 Wh | 4,884 W | 390.7 Wh |
| RTL (1.10× motor + avionics) | 4.0 / 4.0 min | 3,831 W | 255.4 Wh | 5,268 W | 351.2 Wh |
| Landing (0.80× motor + avionics) | 1.0 / 1.0 min | 2,801 W | 46.7 Wh | 3,846 W | 64.1 Wh |
| TOTAL MISSION | 15.1 / 21.8 min | — | 935.6 Wh | — | 1,830.6 Wh |
| RESERVE (from 2,813 Wh usable with voltage sag) | — | — | 1,877.5 Wh (66.7%) | — | 982.6 Wh (34.9%) |

Note:The 36 kg Research Phase provides 66.7% energy reserve versus 34.9% at 45 kg. The generous reserves at both configurations are primarily due to the low nominal pump power of 90W and the high battery capacity from three 12S 30Ah packs. At 36 kg, spray time is reduced proportionally to payload (6.8L vs 15.8L).

7.4 Wind Penalty Analysis

Table 25: Wind Penalty Analysis — Dual Configuration

| Wind Condition | Power Penalty | 36 kg Total / Endurance | 45 kg Total / Endurance | Status |
| --- | --- | --- | --- | --- |
| Calm (0 m/s) | 0% | 3,578 W / ~47.2 min | 4,884 W / ~34.6 min (pure hover); mission: 21.8 min + ~12.1 min reserve | Compliant theoretical |
| Light (2 m/s) | +5% motor | 3,750 W / ~45.0 min | 5,121 W / ~33.0 min | Compliant |
| Moderate (3 m/s) | +10% motor | 3,921 W / ~43.0 min | 5,358 W / ~31.5 min | Compliant with reserve monitoring |
| Strong (5 m/s) | +20% motor | 4,264 W / ~39.6 min | 5,832 W / ~28.9 min; 21.8 min route leaves ~8.8% reserve with 2,400 Wh derated capacity | Marginal at 45 kg |
| Very Strong (7 m/s) | +35% motor | 4,779 W / ~35.3 min | 6,543 W / ~25.8 min; worst-case practical sortie capped near 15 min | Not recommended at 45 kg |

Note: Corrected Hobbywing load data reduces hover endurance versus earlier thrust-scaling estimates. The 45 kg configuration remains within the 20-25 minute theoretical mission requirement in calm conditions, but 5 m/s wind plus 35°C battery derating leaves only about 8.8% reserve for the full 21.8 minute route. Field planning should cap 45 kg sorties to about 18.5 minutes under those conditions unless battery capacity is increased.

7.5 Telemetry Link Budget at 866 MHz

The telemetry link budget is calculated at 866 MHz (center of the 865-867 MHz ISM band in India) using the RFD868x radio module. The link budget is identical for both 36 kg and 45 kg configurations.

Table 26: Telemetry Link Budget at 866 MHz

| Parameter | Value | Notes |
| --- | --- | --- |
| Frequency | 866 MHz | Center of 865-867 MHz ISM band (India) |
| Tx Power | 30 dBm (1W) | RFD868x max output |
| Tx Antenna Gain | 2 dBi | Omni, drone-mounted |
| Rx Antenna Gain | 8 dBi | Yagi, ground station |
| FSPL at 10 km | 111.2 dB | 20log(10) + 20log(866) + 32.44 |
| Received Power | -71.2 dBm | 30 + 2 + 8 - 111.2 |
| Rx Sensitivity | -117 dBm | RFD868x at low data rate |
| Multipath Fading | -15 dB | Two-ray ground reflection over crops |
| Vegetation Attenuation | -5 dB | Dense crop canopy absorption |
| Polarization Mismatch | -3 dB | During roll/pitch maneuvers |
| Receiver Loss | -3 dB | Implementation/connector losses |
| Realistic Link Margin at 10 km | 19.8 dB | Adequate (>15 dB margin); requires 4.5m GCS mast. NOTE: This is the TELEMETRY RADIO LINK RANGE — not the operational mission radius. Practical mission radius at 45 kg with 80% DoD: ~6.8 km round-trip. 10–15 km mission range requires ≥35 Ah battery or relay station (BVLOS certification needed). |
| RFD900x (915 MHz) | BANNED | NOT legal in India |

WARNING: RFD900x (915 MHz) is BANNED in India. Only 865-867 MHz ISM band is legal for telemetry. Use RFD868x exclusively.

7.6 Wire Sizing & Voltage Drop

The wire sizing calculation is presented for both configurations. Current values differ, but voltage drops remain negligible in both cases.

Using the wire resistance formula R = ρL/A, where ρ = 1.72×10⁻⁸ Ω·m for copper, L = 0.3m per pack lead, and A = 8.37×10⁻⁶ m² (8 AWG):

Table 27: Wire Sizing & Voltage Drop — Dual Configuration

| Connection | Length | Area | Resistance | 36 kg Current / ΔV | 45 kg Current / ΔV | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Battery to Busbar (8 AWG) | 0.3m | 8.37 mm² | 0.000616Ω | 26.9A / 0.017V | 36.7A / 0.023V | Negligible |
| Busbar to ESC (10 AWG) | 0.2m | 5.26 mm² | 0.000654Ω | 11.9A / 0.008V | 16.5A / 0.011V | Negligible |
| BEC to Pump (18 AWG) | 0.5m | 0.82 mm² | 0.01049Ω | 7.5A / 0.079V | 7.5A / 0.079V | Acceptable |

7.7 Tank Slosh Dynamics & Baffle Design

Liquid slosh in the spray tank creates dynamic CG shifts that can destabilize the hexacopter during flight manoeuvres. At 36 kg with 6.8L fill, the liquid mass is 18.4% of MTOW; at 45 kg with 15.8L fill, it is 35.6%. The slosh dynamics differ significantly between configurations.

The baffle system divides the tank into 4 compartments using 3 longitudinal HDPE baffles, constraining the liquid free surface to 150mm segments. At 36 kg (6.8L fill, ~44% tank capacity), the effective slosh mass per compartment is approximately 1.75 kg, producing smaller CG excursions. At 45 kg (15.8L fill, full capacity), each compartment holds approximately 4 kg, generating larger slosh forces that the baffles must contain.

Table 28: Slosh Baffle Design Specification

| Parameter | Value | 36 kg Notes | 45 kg Notes | Rationale |
| --- | --- | --- | --- | --- |
| Baffle Material | 3mm HDPE sheet | Same | Same | Chemical resistant, UV stable, weldable |
| Number of Baffles | 3 longitudinal | Same | Same | Creates 4 compartments |
| Baffle Spacing | 150 mm center-to-center | Same | Same | Constrains free surface length |
| Flow-Through Holes | 20mm diameter, staggered | Same | Same | 15% free area per baffle |
| Attachment Method | HDPE extrusion welding | Same | Same | Homogeneous bond to tank wall |
| Top Clearance | 25mm from tank top | Same | Same | Air venting for pressure equalization |
| Bottom Clearance | 15mm from tank bottom | Same | Same | Complete drainage |
| Tank Material | HDPE (rotomolded) | Same | Same | Compatible with all pesticides |
| Compartments | 4 (3 baffles) | ~1.75 kg each | ~4 kg each | Each ~4L capacity |
| Slosh Frequency (baffled) | >5 Hz | ~7 Hz (less mass) | ~5 Hz | Above attitude control bandwidth |

7.8 Pre-Charge Circuit

The pre-charge circuit limits inrush current to the ESC input capacitors when the battery is first connected. Without pre-charge, the capacitors appear as a near-short circuit, potentially welding connector contacts and damaging ESC components. The circuit is identical for both configurations.

Table 29: Pre-Charge Circuit Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Configuration | 2× 25Ω 50W parallel | 12.5Ω effective |
| Peak Current | 50.4V / 12.5Ω = 4.03A | At full charge voltage |
| Peak Power (total) | 50.4² / 12.5Ω = 203W | 101.5W per resistor (2.03× pulse at 0.45s — within wirewound pulse derating). FAILURE MODE: if one resistor opens, second sees 203W — thermal fuse (100°C) must be in series for protection. |
| Time Constant (τ) | 12.5Ω × 0.012 F = 0.15s | 150ms |
| 95% Charge Time (3τ) | 0.45s | 450ms |
| Energy per Cycle | ½ × 0.012 × 50.4² = 15.2 J | Within 50W resistor pulse rating |
| Thermal Fuse | 100°C, 5A | Additional safety protection |

7.9 Busbar Thermal Design

Table 30: Busbar Thermal Design — Dual Configuration

| Parameter | 36 kg Value | 45 kg Value | Notes |
| --- | --- | --- | --- |
| Busbar cross-section | 15× 5 mm = 75 mm² | Same | C110 copper |
| Resistance per meter | 0.000229 Ω/m | Same | ρ/A = 1.72×10⁻⁸ / 75×10⁻⁶ |
| Current at hover | 80.6A | 110A | Total system current |
| Power dissipation | 1.49 W/m | 2.77 W/m | I²R |
| Thermal resistance (conv.) | 15 °C/W/m | Same | Natural convection, no airflow |
| Temperature rise | ~22°C | ~42°C | Above ambient |
| Max temperature (45°C ambient) | ~67°C | ~87°C at hover (45°C ambient). At 50°C ambient + peak climb current (150A): estimated 127°C — EXCEEDS insulation limit. 50 kg hover current is ~128A and must be separately thermal-tested before adoption. | Below 105°C limit at hover (45°C ambient). ⚠ At 50°C ambient + peak climb current (150A): estimated 127°C — EXCEEDS insulation limit. Active cooling or derating required for sustained max-power maneuvers at extreme ambient. |

Note: With corrected Hobbywing load-table currents, the busbar reaches about 67°C at 36 kg and 87°C at 45 kg in 45°C ambient hover. This is below the 105°C insulation limit, but 50°C ambient plus sustained peak climb remains non-compliant without airflow/derating, and the 50 kg upper MTOW case needs separate thermal validation.

7.10 Vibration Analysis

Table 31: Vibration Analysis Summary

| Parameter | Value | Notes |
| --- | --- | --- |
| Arm natural frequency (25mm OD) | 32 Hz | Cantilever beam, 800mm span |
| Shaft rotational frequency at hover | ~32 Hz (36 kg) to ~36 Hz (45 kg) | Official load table RPM: ~1,916 at 36 kg, ~2,146 at 45 kg; shaft frequency overlaps 32 Hz arm mode |
| Blade-pass frequency at hover | ~64-72 Hz | 2-blade propeller; not the primary 32 Hz overlap risk |
| Decision Gate G1 (Ground Sweep) | If vibration > 2g at FC during ground sweep | Gate uses ArduPilot/Pixhawk log criteria: VIBE mostly <30 m/s², no continuously increasing clipping, FFT peak not coincident with arm mode |
| Arm natural frequency (30mm OD) | 48 Hz | Shifts arm mode away from 32-36 Hz shaft rotational excitation; verify by FFT during G1 sweep |
| Vibration damping | No unverified Moongel credit | Damping pads are not credited in analysis until G1 ground sweep proves reduction. If VIBE >30 m/s² or clipping increases, rebalance props/motors and upgrade arms/isolation before flight. |
| IMU isolation | Purmo foam mount for FC | Pixhawk isolation accepted only after log review: VIBE <30 m/s² and Clip0/1/2 not continuously increasing |

Note: Corrected Hobbywing RPM values shift the main risk from blade-pass frequency to shaft rotational frequency: ~32 Hz at 36 kg and ~36 Hz at 45 kg, close to the estimated 32 Hz arm natural frequency. No damping material is credited until Gate G1 log data proves VIBE mostly below 30 m/s² and no continuously increasing IMU clipping.

7.11 EMI/EMC Mitigation (868 MHz Threat Matrix)

Table 32: EMI/EMC Threat Matrix (868 MHz Operations)

| Threat Source | Frequency Range | Severity | Mitigation |
| --- | --- | --- | --- |
| ESC switching noise | 20-200 kHz | High | STP CAN bus; ferrite chokes; separate power/signal grounds |
| Motor commutation | 100-500 Hz | Medium | Twisted motor phase wires; magnetic shielding |
| GPS L1 interference | 1575.42 MHz | Low | GPS mast >200mm above ESC; ground plane |
| RFD868x Tx spurious | 865-867 MHz | Low | Antenna separation >300mm from GPS; shielded RF path |
| Pump motor brush noise | 10-50 kHz | Medium | LC filter on pump supply; twisted pair |
| Cross-coupling (CAN/UART) | Mixed | Medium | STP cables; ferrite chokes at FC end; ground star point |
| EMI validation gate | Bench + tethered full-throttle test | Mandatory before G2: CAN/UART packet errors = 0 during 60s motor run, GPS HDOP change <0.2, compass innovation within ArduPilot limits, RFD868x RSSI/noise logged |  |
| Acceptance status | No flight test data yet | Threat matrix is not proof of compliance. Report now marks EMI/EMC as pending verification, not verified performance. |  |

Note: EMI/EMC threats are configuration-independent since the same electrical system operates in both 36 kg and 45 kg phases. Lower current at 36 kg may slightly reduce ESC switching noise severity, but the mitigation strategy remains identical.

8. Trade-off Analysis

8.1 Motor Selection

Table 33: Motor Selection Trade-Off — Dual Configuration

| Motor | Thrust (12S) | Price | 36 kg TWR | 36 kg SMF TWR | 45 kg TWR | 45 kg SMF TWR | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X9 G2L (Selected) | 24 kg/axis datasheet; 20 kg/axis battery-limited design | ₹7,000/motor | 3.33 | 2.44 Compliant | 2.67 | 1.96 Marginal (Mitigation Mandated) | Selected; TWR calculations use 20 kg/axis until battery-current-limited thrust stand validation |
| X9 Plus G2L (Upgrade) | 28 kg/axis | ₹17,000/motor | 4.67 | 3.42 Compliant | 3.73 | 2.74 Compliant | Full SMF for 45 kg |
| X9 Pro | 22 kg/axis | ₹12,000/motor | 3.67 | 2.69 Compliant | 2.93 | 2.12 Compliant | Marginal (Mitigation Mandated) 45 kg SMF |

Note: At 36 kg, ALL motor options provide SMF TWR above 2.0. The X9 G2L is sufficient for the 36 kg Research Phase. Motor upgrade to X9 Plus G2L is only needed for 45 kg commercial operations requiring SMF TWR > 2.0.

8.2 Flight Controller

Table 34: Flight Controller Trade-Off

| FC | Processor | Price | Notes |
| --- | --- | --- | --- |
| Pixhawk 6C (Selected) | H743, 1GB RAM | ₹18,000 | Proven ArduCopter support; dual IMU |
| Cube Orange+ | H743, 2GB RAM | ₹35,000 | Triple IMU; industrial grade |
| DJI N3 | Proprietary | ₹25,000 | Excellent flight performance; closed source |

8.3 Power Distribution

Table 35: Power Distribution Trade-Off

| Method | Current Rating | Price | Notes |
| --- | --- | --- | --- |
| Copper Busbar (Selected) | 200A | ₹2,500 | Highest reliability; field-serviceable fuses |
| PCB PDB | 100A | ₹1,500 | Compact; limited current; hard to repair |
| XT90 Daisy Chain | 90A per joint | ₹1,000 | Cheapest; weakest; no fuse protection |

9. Conclusion

9.1 Design Achievements

The COEP Agricultural Hexacopter achieves the primary mission objectives when evaluated with battery-limited design thrust and verified component data. At the 36 kg Research Phase, the design achieves TWR 3.33, SMF TWR 2.44, and approximately 47.2 minutes of pure hover endurance. The 10L variant operates at 39.2 kg MTOW with TWR 3.06, SMF TWR 2.24, and approximately 42.0 minutes pure hover. At the 45 kg Operational Phase, TWR is 2.67 with 21.8 minutes theoretical mission endurance and 34.9% nominal energy reserve. The phased 36 kg→39.2 kg→45 kg approach ensures safety systems are validated before full-payload operations.

The updated three-operating-case strategy provides a clearer risk path: 36 kg Research Phase, 39.2 kg 10L tank/fill variant, then 45 kg full 16L tank phase. The 36 kg and 10L cases both keep SMF TWR above 2.0; the 45 kg case remains marginal at 1.96 under the conservative 20 kg/axis battery-limited design thrust and therefore still requires strict G5 verification before any full-payload flight.

9.2 Key Innovations

Phased 36 kg→39.2 kg→45 kg development approach: validates systems at 36 kg, then the required 10L/39.2 kg variant, before full 16L/45 kg operation

Three-option single-motor failure mitigation framework for 45 kg :accept, Y6 coaxial, or X9 Plus G2L upgrade

Regulatory-compliant 868 MHz telemetry using RFD868x :45.8 dB link margin at 10 km

Corrected semi-solid-state battery C-rating of 3C continuous / 5C peak :realistic current budget and safety margins

Comprehensive slosh baffle design with HDPE extrusion welding :4-compartment tank with 15% free-area baffles

Arm resonance decision gate at G1 Ground Sweep :data-driven 30mm arm upgrade trigger

9.3 Future Improvements

14S Battery Migration: Hobbywing datasheet max increases to 25.8 kg/axis at 14S (normal TWR 4.30 at 36 kg / 3.44 at 45 kg before battery-current limits). Requires battery pack, connector, fuse, and thermal redesign; not a drop-in change.

X9 Plus G2L Motor Upgrade: Provides 28 kg/axis at 12S, achieving TWR of 4.67 (36 kg) / 3.73 (45 kg) and SMF TWR of 3.42 / 2.74. Recommended for commercial 45 kg operations.

ADS1256 ADC for Thrust Stand: Enables true 200 Hz sampling for dynamic thrust measurement, replacing HX711 at 80 Hz. Additional cost: ₹1,250 over HX711 option.

Autonomous Swarm Operations: Multiple hexacopters coordinated via ground station for large-area coverage. Requires BVLOS certification and inter-vehicle communication.

AI-Based Spray Optimization: Real-time NDVI camera integration for variable-rate application based on crop health. Reduces chemical usage by 20-40%.

Lithium-Sulfur Battery Integration [Research Concept Only]: At 500 Wh/kg (projected; not commercially available at required capacity as of 2026), reduces battery mass from 14.7 kg to ~7 kg, increasing endurance by 50% or allowing 36 kg MTOW with full 15.8L payload. Not a design commitment for this project.

9.4 Compliance Statement

This design complies with the following regulatory frameworks for research-phase operations within India:

Drone Rules 2021 (amended 2024), MoCA/DGCA: Medium category UAS, MTOW 45 kg (operational) / 36 kg (research) [Note: design was also cross-referenced against draft CAR for future compliance]

WPC Frequency Regulations: 865-867 MHz ISM band for telemetry (RFD868x)

NPNT (No Permission No Takeoff): Module integrated, Digital Sky platform

UIN Registration: FREE on Digital Sky platform

Remote Pilot License: ₹15,000-25,000

Third-Party Liability Insurance: ₹50,000-100,000/year for 45 kg commercial drone

Visual Observer: Mandatory for all VLOS operations (SMF mitigation Option A at 45 kg)

10. References & Datasheets

[1] Hobbywing X9 G2L Motor and ESC Datasheet, Hobbywing Technology Co., Ltd., 2024.

[2] Tattu 30Ah 12S Semi-Solid-State LiPo Battery Datasheet, Grepow Battery Co., 2025.

[3] SHURflo 8000 Series Diaphragm Pump Technical Manual, Pentair, 2023.

[4] RFD868x Telemetry Radio Module Datasheet, RFDesign Pty Ltd, 2024.

[5] Pixhawk 6C Flight Controller Reference Manual, Holybro, 2024.

[6] Ministry of Civil Aviation, Drone Rules 2021, G.S.R. 589(E), as amended by G.S.R. 563(E) (2024). https://digitalsky.dgca.gov.in

[7] WPC Wing, National Frequency Allocation Plan (NFAP) 2024, Ministry of Communications, India.

[8] Indian Council of Agricultural Research (ICAR), Guidelines for Drone-Based Pesticide Application, 2023.

[9] EFT E616P Foldable Hexacopter Frame Assembly Guide, EFT Model, 2024.

[10] TeeJet 11002 Flat-Fan Nozzle Performance Data, Spraying Systems Co., 2023.

[11] ADS1256 24-Bit ADC Datasheet, Texas Instruments, 2024.

[12] HX711 24-Bit ADC Datasheet, AVIA Semiconductor, 2023.

11. Appendices

Appendix A: Complete Bill of Materials

Table 36: Complete Bill of Materials

| # | Component | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| 1 | Hobbywing X9 G2L Motor | 6 | ₹12,000 | ₹72,000 |
| 2 | Hobbywing X9 G2L ESC | 6 | ₹8,000 | ₹48,000 |
| 3 | Hobbywing MFP 36×11 Propeller | 6 | ₹4,000 | ₹24,000 |
| 4 | Tattu 30Ah 12S Semi-Solid-State | 3 | ₹43,624-58,000 | ₹1,30,872-1,74,000 |
| 5 | EFT E616P Frame Kit | 1 | ₹52,000 | ₹52,000 |
| 6 | Pixhawk 6C Flight Controller | 1 | ₹18,000 | ₹18,000 |
| 7 | HGLRC M10 Mini GPS | 2 | ₹2,200 | ₹4,400 |
| 8 | RFD868x Telemetry Radio (868 MHz) | 1 | ₹12,000–28,000 (verify vendor) | ₹12,000 |
| 9 | FrSky R-XSR RC Receiver | 1 | ₹5,500 | ₹5,500 |
| 10 | SHURflo 8000 Diaphragm Pump | 1 | ₹8,500 | ₹8,500 |
| 11 | TeeJet 11002 Nozzle | 4 | ₹1,200 | ₹4,800 |
| 12 | 16L HDPE Tank with Baffles (15.8L operational; 10L variant/fill supported) | 1 | ₹6,500 | ₹6,500 |
| 13 | 15×5mm Copper Busbar | 1 | ₹2,500 | ₹2,500 |
| 14 | MEGA Fuse 150A + Holder | 6 | ₹800 | ₹4,800 |
| 15 | XT90 Anti-Spark Connector | 6 | ₹350 | ₹2,100 |
| 16 | Pre-Charge Resistors (2×25Ω 50W) | 1 set | ₹800 | ₹800 |
| 17 | MAUCH HS-200-LV Current Sensor | 1 | ₹6,000 | ₹6,000 |
| 18 | Parachute Recovery System | 1 | ₹15,000 | ₹15,000 |
| 19 | Anti-Collision Strobe Light | 1 | ₹3,000 | ₹3,000 |
| 20 | NPNT Compliance Module | 1 | ₹30,000 | ₹30,000 |
| 21 | ADS1256 ADC Module (200 Hz option) | 1 | ₹1,500 | ₹1,500 |
| 22 | ESP32-WROOM-32 (Thrust Stand) | 1 | ₹800 | ₹800 |
| 23 | 15A BEC / PMU (avionics + pump) | 2 | ₹1,200 | ₹2,400 |
| 24 | SHURflo Inline Flow Sensor (pulse) | 1 | ₹1,800 | ₹1,800 |
| 25 | Pressure Sensor (0-4 bar, 12V) | 1 | ₹900 | ₹900 |
| 26 | G10 Fiberglass Vibration Isolators (set) | 6 | ₹300 | ₹1,800 |

Note: BOM is identical for both 36 kg and 45 kg configurations. No additional hardware is required for the 36 kg Research Phase.

Appendix B: Vendor Contact Sheet

Table 37: Vendor Contact Sheet

| Vendor | Product | Website | Location |
| --- | --- | --- | --- |
| Hobbywing | Motors, ESCs | hobbywing.com | Shenzhen, China |
| Grepow / Tattu | Batteries | tattu.com | Shenzhen, China / IndiaMART |
| EFT Model | Frame Kit | eftmodel.com | Guangdong, China |
| Holybro | Pixhawk 6C | holybro.com | Shenzhen, China |
| RFDesign | RFD868x Telemetry | rfdesign.com.au | Queensland, Australia |
| SHURflo / Pentair | Diaphragm Pump | pentair.com | USA / India Distributor |
| Spraying Systems | TeeJet Nozzles | spray.com | USA / India Distributor |

Appendix C: DGCA Compliance

C.1 Research Phase

Table 38: DGCA Compliance — Research Phase (36 kg / 45 kg)

| Item | Cost | 36 kg Notes | 45 kg Notes |
| --- | --- | --- | --- |
| UIN Registration | FREE | Digital Sky platform | Same |
| NPNT Module | ₹20,000-40,000 | Hardware + integration | Same |
| Remote Pilot License (RPL) | ₹15,000-25,000 | DGCA-approved training | Same |
| Third-Party Insurance | ₹50,000-100,000/year | Applicable to both configs | Mandatory for all operations, including research |
| TOTAL (Research Phase) | ₹1,05,000–2,05,000 (incl. NPNT ₹20,000–40,000) | Annual recurring: insurance + RPL | Same |

C.2 Commercial Phase

Table 39: DGCA Compliance — Commercial Phase

| Item | Cost | Notes |
| --- | --- | --- |
| Type Certificate | ₹5,00,000+ | DGCA type certification process |
| Production Organization Approval | ₹2,00,000+ | Manufacturing quality system |
| Continued Airworthiness | ₹1,00,000/year | Ongoing compliance |
| TOTAL (Commercial Phase) | ₹8,00,000+ | One-time + annual recurring |

Appendix D: Thrust Stand BOM

Table 40: Thrust Stand BOM

| Component | Qty | Price |
| --- | --- | --- |
| 50kg S-type Load Cell (YZC-516) | 1 | ₹1,200 |
| ADS1256 24-bit ADC Module (30 kHz, 200 Hz) | 1 | ₹1,500 |
| HX711 24-bit ADC (80 Hz max, budget option) | 1 | ₹250 |
| ESP32-WROOM-32 | 1 | ₹800 |
| ACS758-200B Current Sensor | 1 | ₹600 |
| I-Beam Test Rig | 1 | ₹2,500 |
| XT90-S + 10 AWG Wire | 1 set | ₹300 |
| Micro SD + 16GB Card | 1 | ₹350 |
| Total (ADS1256 Option) | — | ₹7,250 |
| Total (HX711 Budget Option) | — | ₹6,000 |

Note:The HX711 ADC delivers a maximum of 80 SPS (samples per second). For dynamic thrust measurement requiring 200 Hz, the ADS1256 ADC (30 kHz, true 200 Hz sampling) is recommended. The ESP32-WROOM-32 controller replaces the Arduino Nano due to insufficient processing speed on the Nano for real-time 200 Hz data logging.

Appendix E: Motor Datasheet

Table 41: Hobbywing X9 G2L Load Performance Parameters (12S, MFP 36x11)

| Prop(inch) | Throttle | Voltage(v) | Amps(A) | Thrust(gf) | Watts(W) | Efficiency(g/W) | RPM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MFP 36×11 | 33% | 48.0 V | 7.8 A | 4,353 gf | 375.6 W | 11.6 g/W | 1,637 RPM |
| MFP 36×11 | 39% | 48.0 V | 11.6 A | 5,884 gf | 556.0 W | 10.6 g/W | 1,898 RPM |
| MFP 36×11 | 45% | 48.0 V | 16.6 A | 7,545 gf | 796.6 W | 9.5 g/W | 2,153 RPM |
| MFP 36×11 | 51% | 48.0 V | 22.5 A | 9,284 gf | 1,079.9 W | 8.6 g/W | 2,392 RPM |
| MFP 36×11 | 60% | 48.0 V | 32.7 A | 12,009 gf | 1,570.0 W | 7.6 g/W | 2,725 RPM |
| MFP 36×11 | 72% | 48.0 V | 49.5 A | 15,867 gf | 2,377.3 W | 6.7 g/W | 3,148 RPM |
| MFP 36×11 | 84% | 48.0 V | 70.9 A | 19,859 gf | 3,405.3 W | 5.8 g/W | 3,570 RPM |
| MFP 36×11 | 100% | 48.0 V | 95.9 A | 23,997 gf | 4,607.3 W | 5.2 g/W | 4,000 RPM |

Table 47: Hobbywing X9 G2L Load Performance Parameters (12S, MFP 36x11)

Appendix F: Weather Data

Table 42: Atmospheric Weather Profile vs. Altitude Above Ground Level

| Above ground level(ft) | Temperature (℃) | Pressure(lb/ft2) | Pressure (mm of Hg) | Density (SLUGS/Ft³) |
| --- | --- | --- | --- | --- |
| 0 ft | 15.0 ℃ | 2,116.2 lb/ft² | 760.0 mm of Hg | 0.0023769 SLUGS/Ft³ |
| 500 ft | 14.0 ℃ | 2,078.5 lb/ft² | 746.4 mm of Hg | 0.0023425 SLUGS/Ft³ |
| 1,000 ft | 13.0 ℃ | 2,041.4 lb/ft² | 733.1 mm of Hg | 0.0023085 SLUGS/Ft³ |
| 1,500 ft | 12.0 ℃ | 2,004.8 lb/ft² | 720.0 mm of Hg | 0.0022748 SLUGS/Ft³ |
| 2,000 ft | 11.0 ℃ | 1,968.7 lb/ft² | 707.0 mm of Hg | 0.0022415 SLUGS/Ft³ |
| 3,000 ft | 9.1 ℃ | 1,898.1 lb/ft² | 681.7 mm of Hg | 0.0021760 SLUGS/Ft³ |
| 4,000 ft | 7.1 ℃ | 1,829.4 lb/ft² | 657.0 mm of Hg | 0.0021118 SLUGS/Ft³ |
| 5,000 ft | 5.1 ℃ | 1,762.4 lb/ft² | 632.9 mm of Hg | 0.0020490 SLUGS/Ft³ |

Table 48: Atmospheric Weather Profile vs. Altitude Above Ground Level

Appendix G: Real-Value Verification Addendum

This addendum closes the remaining v7 open items using datasheet values and mandatory validation gates. Sources checked include Hobbywing X9 G2L official product/load table, Grepow/Tattu 12S 30Ah semi-solid-state battery page, TeeJet XR11002 published nozzle rating, ArduPilot vibration guidance, RFD868x datasheet/search result summary, Pixhawk 6C public documentation, and YF-S402 flow-sensor product data. Values that still require hardware testing are explicitly marked as verification gates rather than claimed measured performance.

| Item | Verified Value | Report Action | Gate | Source Basis |
| --- | --- | --- | --- | --- |
| X9 G2L propulsion | 24 kg/axis datasheet max at 12S; 20 kg/axis battery-limited design thrust | TWR keeps conservative 20 kg/axis until current-limited thrust test | G1 thrust stand | Hobbywing X9 G2L official load table |
| Tattu 12S 30Ah | 44.4V, 30Ah, 1,332Wh, 3C continuous/90A, 5C burst/150A, 4.9kg | Battery burst cap explains thrust limit | Battery current log | Grepow/Tattu datasheet page |
| 10L variant | MTOW 39.2 kg; TWR 3.06; SMF TWR 2.24; pure hover ~42.0 min | Added as required intermediate configuration | G4 before 45 kg | Calculated from dry mass + 10L water |
| Flight time | 45 kg theoretical mission 21.8 min; conservative planning 18.5 min at 35°C/5 m/s | Table 24 consistency fixed | Mission log | Corrected Hobbywing power + Tattu usable energy |
| Nozzle flow | XR11002: 0.20 gpm at 40 psi; 4 nozzles ~3.03 L/min at 40 psi, ~2.65 L/min at 2.1 bar | Removed incorrect 3.16 L/min at 2.0 bar claim | Bucket test | TeeJet XR11002 published rating |
| Nozzle wear | Replace when flow exceeds new-nozzle baseline by >10% at same pressure | Maintenance criterion added | Every 25 tank cycles | TeeJet/ag sprayer standard practice |
| Flow sensor | YF-S402 0.3-6 L/min, f=(73×Q)±2%, 4380 pulses/L | Calibration table/procedure added | G2 and weekly | YF-S402 product data |
| Vibration | ArduPilot acceptable VIBE mostly <30 m/s²; >60 m/s² problematic; clipping must not increase | Moongel not credited without test | G1 ground sweep | ArduPilot vibration guidance |
| EMI/EMC | No measured data yet | Threat matrix marked pending; acceptance criteria added | G2 full-throttle tether | ArduPilot/Pixhawk logging practice |

Appendix H: Flow Calibration Procedure

Before Gate G2, run clean water through the final tank, filter, pump, flow sensor, pressure gauge, and nozzle assembly. Collect discharge into a graduated container for 60 seconds at four set points: 1, 2, 3, and 5 L/min command. Record pulses, measured volume, pressure, and battery voltage. Compute K-factor as pulses per liter; initial YF-S402 estimate is 4380 pulses/L. Flight controller calibration is accepted only if corrected volume error is within ±3% at 1-3 L/min and within ±5% at 5 L/min. Recalibrate after nozzle replacement, chemical change, pump service, or every 25 tank cycles.

Appendix I: Conservative Flight Envelope

| Scenario | Usable Energy | Weather/Wind | 45 kg Mission Result | Planning Decision |
| --- | --- | --- | --- | --- |
| Nominal | 2,813 Wh | 25°C, calm | 21.8 min mission, 34.9% reserve | Compliant theoretical 20-25 min |
| Conservative Indian summer | 2,400 Wh | 35°C, 5 m/s wind | 21.8 min route leaves ~8.8% reserve | Plan 18.5 min to preserve ~20% reserve |
| Worst-case hot/windy | 2,000 Wh | 45°C, 7 m/s wind | 21.8 min route not energy-compliant | Cap at ~15 min or reduce payload |